# Supremelogic Limited

New House, Church Road, Llanishen, Chepstow, Monmouthshire NP16 6QS, UK

Tel. +44(0)1600860117		E-mail address: anis@supremelogic.co.uk

Fax +44(0)1600 869075

ANALYSIS Software

for

DOS and Windows

INTRODUCTION

After releasing TUNNEL1 and TUNNEL2 to University of Wales College of Cardiff (UWCC) at the end of 1994, four years were spent thenceforth in the development of ANALYSIS software that runs under DOS and Windows. It is a new program that includes all the capabilities of the previous programs among its numerous other capabilities and has unique outstanding presentation. These computer programs were the result of colossal efforts and intensive research. They implement high technology and original solutions to the most difficult structural engineering problems that were not hitherto available.

SCOPE OF PROGRAM

Here below is a brief description of the most prominent capabilities of ANALYSIS software

1 – Determination of the buoyancy of immersed sections.

2 – Determination by the finite element analysis method of nodal internal forces and

displacements of elastic structures that are idealised as an assemblage of discrete

beam-column elements, supported by self-equilibrating external forces and

pressures. The formulation incorporates deformations due to bending, shear and

axial forces, the non-linear influence of axial forces on bending and equivalent

nodal force vectors corresponding to trapezoidal loading, uniformly distributed

loading and linear thermal gradients across elements. Elastic subgrade foundation

elements are incorporated in the analysis of foundation structures.

3 -  Plotting the nodal internal moment and shear diagrams.

4 -  Design of shear connectors for double skin composite (DSC -  to resist

compression plate buckling, interface shearing forces and to act as transverse

shear reinforcement.

5 -  Determination of ultimate moment of resistance of DSC sections subject to

bending and axial forces.

6 -  Design for shear reinforcement of RC beam-column elements.

7 -  Determination of ultimate moment resistance of reinforced concrete RC – sections

subject to bending and axial forces.

8 -  Disply and or print literature, axes, input  forms, and sections' details.

9 -  Provide a drafting board for plotting of figures and cross-sections.

FORMULATION

The program has been originally developed as a general-purpose program to perform all the calculations implied by the finite element formulation. The analysis can be applied to any plane assemblage of discrete beam column elements. The program determines the internal forces (axial, bending and transverse shear) acting within the plane of the cross-section, which is idealized as an assemblage of discrete beam - column, finite elements (see Figure 5.2). The analysis incorporates deformations due to bending, axial and transverse shearing forces, the non-linear influence of axial forces on bending, equivalent nodal force vectors corresponding to trapezoidal and uniformly distributed loading, and linear thermal gradients across elements (5). The program also incorporates elastic foundation elements to model the effect of inter-granular soil pressures beneath the foundations and designs RC and DSC elements. The design of RC elements is based on ACI Code. The design of DSC elements is based on Euro Code and on research in the School of Engineering, UWCC.

The program has been validated, by comparing the numerical results for a carefully chosen range of problems, with available analytical solutions.

OPTIONS OFFERED BY ANALYSIS SOFTWARE

1)	The use of mouse or soft keys menu to control program execution during the 	entire performance in DOS environment

2)	Graphical I/O capability is added to create dialogs and widgets for effective graphical user interfaces to control program execution during the entire performance in Windows environment.

3)	Determination of nodal displacements, internal forces and moments

4)	Designing double skin composite elements

5)	Designing reinforced concrete elements

6)	Checking the design of double skin composite elements against ACI Code

7)	The use of drafting board for accurate and complex graphics work

INPUT OF DATA

ANALYSIS Program displays on the screen at proper intervals all the data input forms that are then filled by the user with the required information. Corrections of the data input can be made at all times, and printing the Data Input Forms is an available option.  For demonstration, the data of the selected example is entered by DEMO.EXE program on behalf of the user.

PROGRAMMING LANGUAGES

The programming language of the original programs developed during the author’s Ph.D. work, were Microsoft BASIC 7.1 for TUNNEL1 and Hewlett-Packard Rocky Mountain BASIC 3.0 for HP9000 Series 200 Workstations for TUNNEL2 programs.

The programming language of the present development is HP BASIC for Windows and HP BASIC PLUS. ANALYSIS program can run under DOS 5.0 or above and under Windows 95+ or Windows 3.1x (with Win32s installed).

SYSTEM REQUIREMENTS

A 386/486/Pentium class processor or better is required. A processor with built-in numeric processor (486, Pentium) or an external numeric coprocessor (387SX, 387DX, 487) is strongly recommended. Four megabytes of free RAM memory are required, but 8 megabytes are recommended. If memory is inadequate, the program displays the free available memory and gives proper advice. The program will run with a monochrome, 16-colour, 256-colour or 24-bit display driver, but a 256-colour display driver is recommended. COLOR MAP functions only with 256-color display driver. The program can send screen dumps to any graphic printer supported by Windows. Plotters must be HPGL or PostScript compatible.

ENVIRONMENT

ANALYSIS runs in DOS and Windows as follows

- DOS version –

It runs in MS-DOS version 5.0 or above. Microsoft mouse software must be installed before running the program.

2)	Windows version-

a.	Windows 3.1x - Add Microsoft WIN32S to the system.

b.	Windows 95+.  Set monitor colors to 256.

THE DEMONSTRATION PROGRAM

To demonstrate the performance of ANALYSIS, a Demo program that presents the various features and capabilities of the software has been developed. It solves the same example presented in TUNNEL2 Users’ Guide (parts thereof are included in this file).

Although the intended use of the master program is for much more complicated problems, this example was carefully selected simple to allow checking the computer output by hand calculations. The Demo diskette is easy to run and pleasant to watch.

RUNNING THE DEMO PROGRAM

Before running the Demo program, make sure that the computer complies with the limitations stated under SYSTEM REQUIREMENTS and ENVIRONMENT mentioned above. After that is done, insert the Demo diskette in drive A, keep it inserted during the entire performance of the program, and strictly heed the instructions displayed on the screen by the Demo program.

If Windows version is used, invoke DEMO.EXE by the File Manager (Windows Explorer in Windows 95). The Demo program runs for 1 hour then quits and returns control to Windows.

If MS-DOS version is used, type A:\>demo then press [ENTER]. The Demo program runs for 1/2 hour then quits and returns control to DOS drive A.

To re-run either Demo program repeat the relevant steps explained above.

QUITTING

The program allows quitting at certain intervals by pressing [Esc]. Quitting at other times may be achieved by pressing Alt + F4. This halts program execution. To complete quitting type quit then press [ENTER]. Completion of quitting in Windows version can also be achieved by clicking the mouse on the top left corner. This pops up a dialogue box. Clicking the mouse once more on Close completes quitting.

RE-RUNNING AFTER A HALT

The program may be re-run after a halt by Press Alt + F3. The same effect can be achieved by typing run followed by pressing [ENTER].

INTERNET REFERENCE

To view the Author's address and software specifications, please visit the following web site

http://homepage.ntlworld.com/a.helou/Neue_Dateien/

Dr. Anis J. Helou

# Managing Director

EXAMPLE OF PROGRAM APPLICATION

5.1	INTRODUCTION

An illustrative example of the application of the program for the design of a single bore DSC immersed tube tunnel, is presented in this chapter. Preparation of input data is discussed in detail in chapter 3. The chosen units of measurement are kilonewtons and meters, abbreviated as kN and m.

5.2	PREPARATION OF INPUT DATA

5.2.1	Buoyancy

Figure 5.1 shows the assumed tunnel cross-section, with the exterior, interior and ballast surfaces defined by suitable elements and nodes. The origin of the global co-ordinates is at node 1 so that all nodal co-ordinates are positive. Information contained in Figure 5.1 and the following data, was used to complete input data forms 1 to 3.

Total number of surface elements			12

Total number of nodes				10

Average steel plate thickness

Interior						0.012 m

Exterior					0.012 m

Unit weight of materials

Concrete					24 kN/m3

Ballast						22 kN/m3

Water						10 kN/m3

5.2.2	Finite element representation

Figure 5.2 shows the finite element representation of the tunnel cross-section. Information contained in Figure 5.2 and the following data, was used to complete input data forms, 4 and 5.

Total number of elements			   	9

Total number of nodes				6

Total number of elastic foundation elements		3

Elastic foundation modulus				40,000 kN/m2/m

Modulus of elasticity of structural elements		2x107 kN/m2

Poison's ratio of structural elements			0.2

Second moment of area of structural elements	0.083333 m4

Cross-sectional area of structural elements		1.00 m2

Unit weight of elements 1 to 3			27 kN/m

Unit weight of elements 4 to 6 plus ballast 		45 kN/m

5.2.3	Loading

Figure 5.3 shows the assumed trapezoidal and concentrated loading on the cross-section and the assumed external and internal temperatures. The information contained in Figure 5.3 was used to complete input data forms 6 to 9. Since there are no assumed fixed boundary conditions, input data form10 does not have to be completed.

5.2.4	Design of shear connection

The assumption of various material properties and the previously specified steel plate thickness and concrete depths enables completion of input data forms 11 and 12.

5.2.5	Design for ultimate moment and axial force

The assumption of various material properties, the previously specified steel plate thickness and concrete depth, and the information output by the program in Table 12, enables completion of input data form13.

5.3	RUNNING THE PROGRAM

5.3.1	General

Printing the output Tables and dumping the Figures are available options during the first and subsequent runs of the program. Since most of the input and output data is recorded automatically in binary data files on the MSM, printing Tables and dumping Figures may, at the discretion of the user, be postponed to a subsequent run.

5.3.2	Buoyancy

The input data in forms 1 to 3 was entered into the computer memory during the first run. Executing the buoyancy subroutine was carried out in accordance with the procedures presented in Sections 4.2.1, 4.2.2 and 4.3.

The computer output is shown in Tables 1 to 3, which are also recorded as binary data files on the MSM.

5.3.3	Finite element analysis

The input data forms, 4 to 8 were entered into the computer memory during the first run. Executing the finite element routine was carried out in accordance with the procedures presented in Sections 4.2.1, 4.2.2 and 4.4.

The computer output is shown in Tables 4 to 11 and Figures 2 and 3. Tables 4 to 10 are recorded as binary data files on the MSM. Table 11 is computed each time the subroutine is executed.

5.3.4	Design of shear connection

The input data forms, 10 and 11 was entered into the computer memory during the first run. Executing the design subroutine was carried out in accordance with the procedures presented in Sections 4.2.1, 4.2.2 and 4.5.

The computer output is shown in Tables 13 and 14. Table 13 is recorded as binary data file on the MSM. Table 14 is computed each time the program is executed.

5.3.5	Design for ultimate moment and axial force

The data in form 12 and the internal bending moment and axial compressive force in output Table 11 (node 1 element 1) was entered into the computer memory. Executing the design subprogram was carried out in accordance with the procedures presented in Sections 4.2.1, 4.2.2 and 4.6.

The computer output is shown in Figure 5. No binary data files are created on the MSM. Therefore, dumping of the graphics, if required, should be performed during each run.

*********************************************************************